Assignment Solutions: Advanced IP Subnetting Scenarios

This technical report details the subnetting solutions for the provided scenarios. As a Senior Network Architect, I have designed these addressing schemes to ensure high efficiency in address space utilisation, manageable broadcast domains, and robust security segmentation across the various organisational departments.

--------------------------------------------------------------------------------

1. Part 1: Class C Subnetting Scenarios

1.1. Scenario 1: The Small Law Firm (192.168.15.0/24)

The Situation: A law firm requires exactly 4 subnets to segment traffic for Partners, Associates, Paralegals, and Guest Wi-Fi.

- Task A Solution: To create exactly 4 subnets, we must determine the number of borrowed bits (n) from the host portion of the /24 mask. 2^2 = 4 By borrowing 2 bits, we create the required subnets.

  - New Mask (CIDR): /26

  - New Mask (Decimal): 255.255.255.192

- Task B Solution: The number of usable host IP addresses per subnet is calculated by the formula 2^h - 2, where h is the number of remaining host bits (8 - 2 = 6). 2^6 - 2 = 62 Each subnet provides 62 usable hosts.

- Task C Solution: The configuration for the 2nd subnet (Associates) is as follows:

  - Network ID: 192.168.15.64

  - Usable IP Range: 192.168.15.65 - 192.168.15.126

  - Broadcast Address: 192.168.15.127

1.2. Scenario 2: The Medical Clinic (200.50.25.0/24)

The Situation: The clinic requires a minimum of 6 subnets to isolate departments such as Reception, Doctors, and Pharmacy for regulatory compliance.

- Task A Solution: To accommodate at least 6 subnets, we find the smallest power of 2 that is greater than or equal to 6. 2^3 = 8 Borrowing 3 bits satisfies the requirement and provides a total of 8 subnets.

- Task B Solution:

  - New Mask (CIDR): /27

  - New Mask (Decimal): 255.255.255.224

- Task C Solution: For the 1st subnet (Reception):

  - Network ID: 200.50.25.0

  - Broadcast Address: 200.50.25.31

1.3. Scenario 3: The High School Computer Labs (192.168.100.0/24)

The Situation: The school requires 4 subnets, each supporting a minimum of 50 computers.

- Task A Solution: To support 4 labs, we borrow 2 bits (2^2 = 4 subnets). We must verify that the remaining 6 host bits meet the 50-host requirement: 2^6 - 2 = 62 hosts. Choosing a /25 mask would provide only 2 subnets (failing the lab count requirement), while a /27 mask would only provide 30 usable hosts (failing the capacity requirement).

  - New Mask (CIDR): /26

  - New Mask (Decimal): 255.255.255.192

- Task B Solution: This specific mask provides a maximum of 62 usable hosts per subnet.

- Task C Solution: The Usable IP Range for the 4th subnet (incrementing by 64) is 192.168.100.193 - 192.168.100.254.

1.4. Scenario 4: The Coffee Shop Chain (198.100.50.0/24)

The Situation: The goal is to maximise the number of subnets while ensuring at least 14 usable hosts per subnet for point-of-sale systems.

- Task A Solution: We prioritise the host requirement. To support 14 hosts, we need h bits where 2^h - 2 \ge 14, which results in h = 4 (2^4 = 16). This leaves 4 bits to be borrowed from the original /24. A /27 mask would provide 30 hosts but only 8 subnets, whereas a /29 mask would only provide 6 usable hosts, failing the requirement.

  - New Mask (CIDR): /28

  - New Mask (Decimal): 255.255.255.240

- Task B Solution: Total subnets created: 2^4 = 16 This configuration results in 16 subnets.

- Task C Solution: Using an increment (magic number) of 16, the Network ID for the 3rd subnet is 198.100.50.32.

--------------------------------------------------------------------------------

2. Part 2: Class B Subnetting Scenarios

2.1. Scenario 6: The Regional Hospital (172.16.0.0/16)

The Situation: The hospital needs 50 separate subnets to isolate floors and life-support machine networks.

- Task A Solution: To create at least 50 subnets, we borrow bits from the 3rd octet. 2^6 = 64 Borrowing 6 bits provides 64 subnets, satisfying the requirement.

  - New Mask (CIDR): /22

  - New Mask (Decimal): 255.255.252.0

- Task B Solution: With 10 bits remaining for hosts (32 - 22 = 10): 2^{10} - 2 = 1022 Each subnet supports 1022 usable hosts.

- Task C Solution: Using a magic number increment of 4 in the 3rd octet, the 2nd subnet details are:

  - Network ID: 172.16.4.0

  - Broadcast Address: 172.16.7.255

2.2. Scenario 7: The Corporate Headquarters (172.25.0.0/16)

The Situation: Subnets must be sized to support exactly 500 usable host addresses to allow for future employee growth.

- Task A Solution: To support at least 500 hosts, we find h host bits: 2^9 - 2 = 510 We require 9 host bits.

- Task B Solution: Subtracting 9 host bits from 32 total bits gives us a 23-bit network prefix.

  - New Mask (CIDR): /23

  - New Mask (Decimal): 255.255.254.0

- Task C Solution: The "Block Size" (magic number) in the 3rd octet for these subnets is 2.

2.3. Scenario 8: The Smart City Project (128.10.0.0/16)

The Situation: 100 subnets are needed for city grids, infrastructure, and public Wi-Fi zones.

- Task A Solution: To create at least 100 subnets: 2^7 = 128 We must borrow 7 bits.

  - New Mask (CIDR): /23

  - New Mask (Decimal): 255.255.254.0

- Task B Solution: With 9 bits remaining for hosts: 2^9 - 2 = 510 There are 510 usable IP addresses available in each city grid subnet.

- Task C Solution: The Network ID for the 3rd subnet (incrementing by 2 in the 3rd octet) is 128.10.4.0.

2.4. Scenario 9: The University Wi-Fi Zones (172.31.0.0/16)

The Situation: Each wireless zone must support 2,000 devices simultaneously.

- Task A Solution: To support 2,000 hosts: 2^{11} - 2 = 2046 We require 11 host bits.

  - New Mask (CIDR): /21

  - New Mask (Decimal): 255.255.248.0

- Task B Solution: The number of subnets created from the original /16 is determined by the 5 bits borrowed (21 - 16 = 5). 2^5 = 32 The university can create 32 subnets.

- Task C Solution: The Usable IP Range for the 1st subnet (block size of 8 in the 3rd octet) is 172.31.0.1 - 172.31.7.254.

2.5. Scenario 10: The Manufacturing Plant (150.20.0.0/16)

The Situation: The network must be divided into exactly 16 equal-sized subnets for assembly line isolation.

- Task A Solution: To create exactly 16 subnets: 2^4 = 16 We must borrow 4 bits.

  - New Mask (CIDR): /20

  - New Mask (Decimal): 255.255.240.0

- Task B Solution: With 12 host bits remaining (32 - 20 = 12): 2^{12} - 2 = 4094 There are 4094 usable hosts available per assembly line.

- Task C Solution: The details for the 5th subnet (using an increment of 16 in the 3rd octet) are:

  - Network ID: 150.20.64.0

  - Usable IP Range: 150.20.64.1 - 150.20.79.254

  - Broadcast Address: 150.20.79.255

--------------------------------------------------------------------------------

3. Summary Table of Calculated Masks

| Scenario Number | Original Block | Required Subnets/Hosts | New Mask (CIDR) | New Mask (Decimal) |
| --- | --- | --- | --- | --- |
| 1 | 192.168.15.0/24 | Exactly 4 Subnets | /26 | 255.255.255.192 |
| 2 | 200.50.25.0/24 | Min 6 Subnets | /27 | 255.255.255.224 |
| 3 | 192.168.100.0/24 | 4 Subnets / 50 Hosts | /26 | 255.255.255.192 |
| 4 | 198.100.50.0/24 | Max Subnets / 14 Hosts | /28 | 255.255.255.240 |
| 6 | 172.16.0.0/16 | Min 50 Subnets | /22 | 255.255.252.0 |
| 7 | 172.25.0.0/16 | 500 Usable Hosts | /23 | 255.255.254.0 |
| 8 | 128.10.0.0/16 | 100 Subnets | /23 | 255.255.254.0 |
| 9 | 172.31.0.0/16 | 2,000 Usable Hosts | /21 | 255.255.248.0 |
| 10 | 150.20.0.0/16 | Exactly 16 Subnets | /20 | 255.255.240.0 |